DSL Project

MorseML (STM32F429)

Basically, this project listens to you if you talk to it in Morse code and subsequently decipher whatever you say! All you need is a Push Button (Blue Button on STM32 board).

This project decodes Morse code using:

- STM32F429 (bare-metal programming, no HAL)

- TensorFlow Lite Micro (embedded CNN)

- USART (PuTTY output)

The system captures button press timings, converts them into a feature vector, and uses a trained CNN model to predict the corresponding alphabet.

What is Morse Code?

Morse code is a method of encoding text using:

- Short signals → DOT (·)

- Long signals → DASH (–)

These are represented using:

- Button press duration → DOT or DASH

- Gap between presses → separation between symbols

Morse Representation of Supported Letters

| Letter | Morse | Pattern |
| --- | --- | --- |
| A | · – | short, long |
| B | – · · · | long, short, short, short |
| C | – · – · | long, short, long, short |
| D | – · · | long, short, short |
| E | · | short |

How STM32 Interprets Morse

The STM32 does not directly detect dots and dashes.

Instead, it measures:

- Press duration → press[i]

- Gap duration → gap[i]

Example:

A (· –):

Press1 ≈ 100ms

Gap1 ≈ 100ms

Press2 ≈ 300ms

Converted into feature vector:

[100, 100, 300, 0, 0]

How ML Model Uses Morse

The ML model does not see symbols (· or –).

It only sees:

[press1, gap1, press2, gap2, press3]

This makes the system robust to:

- Different typing speeds

- Slight human errors

How to Add Morse to the Model

Step 1: Define Pattern

Each letter must be converted into a numerical pattern:

A → [100,100,300,0,0]

B → [300,100,100,100,100]

C → [300,100,300,100,0]

D → [300,100,100,0,0]

E → [100,0,0,0,0]

Step 2: Generate Dataset

Run:

python dataset_generator.py

This:

- Adds noise (±10ms)

- Creates multiple samples per letter

- Improves generalization

Step 3: Train Model

python train_cnn.py

This trains a CNN classifier that maps:

timing → letter

Step 4: Convert to Embedded Model

python convert_to_tflite.py

xxd -i model.tflite > model_data.h

How STM32 Uses This

- User presses button in Morse

- STM32 records timings

- Converts to:

[press1, press2, press3, ...]

- Normalizes:

input_data[i] = press[i] / 1000.0;

- Feeds into CNN

- Outputs:

Predicted: A

Important Design Decisions

Why not detect dots and dashes manually?

Because:

- Humans are inconsistent

- Timing varies

- Hard thresholds fail

Why ML?

- Learns patterns automatically

- More robust to variation

- Easier to extend

Improving Accuracy

1. Add More Data

Increase dataset:

for _ in range(1000):

2. Add More Noise

Simulate real users:

noise = np.random.normal(0, 20, 5)

3. Normalize Better

Instead of absolute time:

X = X / np.max(X, axis=1, keepdims=True)

4. Add Gap Features (Advanced)

Currently only press is used.

Better:

[press1, gap1, press2, gap2, press3]

Hardware Setup

- Board: STM32F429ZITx

- Button: PC13 (User Button)

- USART1:

  - PA9 → TX → PuTTY

  - PA10 → RX (unused)

PuTTY Configuration

- Baud Rate: 115200

- Data Bits: 8

- Stop Bits: 1

Example Output

Morse CNN Ready

Predicted: A

Predicted: B

Predicted: E

Future Improvements

- Add full alphabet A–Z

- Add word decoding

- Add LCD output

- Real-time signal graph

- BLE or WiFi input

Summary

This project demonstrates:

- Embedded systems (bare-metal STM32)

- Signal processing (timing extraction)

- Machine Learning (CNN)

- Edge AI deployment (TFLite Micro)

A complete embedded and AI system.